HOME CHECK LIST

MONTHLY

● Extinguisher: Check that it’s fully charged; recharge or replace if needed.

● Sink/Tub Stoppers and Drain Holes: Clean out debris.

● Garbage Disposal: Flush with hot water and baking soda.

● Water Softener: Check water softener salt drum and replenish salt if necessary.

● Forced-Air Heating System: Change filters once a month if user’s manual recommends fiberglass      filters.

EVERY 2 MONTHS

● Wall Furnace: Clean grills.

● Range Hood: Clean grease filter

EVERY 3 MONTHS

● Faucet: Clean aerator.

● Tub Drain Assembly: Clean out debris; inspect rubber seal and replace if needed.

● Floor and Outdoor Drain Grates: Clean out debris.

EVERY 6 MONTHS

● Smoke Detector: Test batteries and replace if needed.

● Toilet: Check for leaks and water run-on.

● Interior Caulking: Inspect caulking around tubs, showers, and sinks; replace any if it is deteriorating.

● Forced-Air Heating System: Change semi-annually if user’s manual recommends high efficiency pleated or HEPA-style filters.

● Garbage Disposal: Tighten drain connections and fasteners.

● Clothes Washer: Clean water inlet filters; check hoses and replace them if they are leaking.

● Clothes Dryer: Vacuum lint from ducts and surrounding areas.

● Wiring: Check for frayed cords and wires; repair or replace them as needed.

● Range Hood: Wash fan blades and housing.

List courtesy of Linda Gemal, Rejuvenate Your Home (www.rejuvenateyourhome.com.)

HOME CHECK LIST – PAGE 2

EVERY SPRING

● Roof: Inspect roof surface, flashing, eaves, and soffits; repair if needed.

● Gutters and Downspouts: Clean them out or install no-clean version. Inspect and repair weak areas; check for proper drainage and make repairs if needed.

● Siding: Inspect and clean siding and repair if needed.

● Exterior Caulking: Inspect caulking and replace any that is deteriorating.

● Windowsills, Doorsills, Thresholds: Fill cracks, caulk edges, repaint; replace if needed.

● Window and Door Screens: Clean screening and repair or replace if needed; tighten or repair any loose or damaged frames and repaint if needed; replace broken, worn, or missing hardware; tighten and lubricate door hinges and closers.

EVERY FALL

● Roof: Inspect roof surface, flashing, eaves, and soffits; repair if needed.

● Gutters and Downspouts: Clean out. Inspect and repair weak points; check for proper slope.

● Chimney or Stovepipe: Clean flue (more frequently if needed); repair any cracks in flue or any loose or crumbling mortar.

● Siding: Inspect and clean siding and repair if needed.

● Exterior Caulking: Inspect caulking and replace any that is deteriorating.

● Storm Windows and Doors: Replace any cracked or broken glass; tighten or repair any loose or damaged frames and repaint if needed. Replace damaged hardware; tighten and lubricate door hinges  and closers.

● Window and Door Weather Stripping: Inspect and repair or replace if it is deteriorating or if it does not seal.

● Thermostat: Clean heat sensor, contact points, and contacts; check accuracy and replace thermostat if it is not functioning properly.

● Outdoor Faucets: If you live in an area with freezing winters, shut off valves to outdoor faucets. Open spigots and drain, store hoses.

ANNUALLY

● Septic Tank: Have a professional check the tank (watch for backup throughout the year). In many areas, it is recommended that the tank be pumped every year.

● Main Cleanout Drain: Have a “rooter” professional clean out the main line, particularly if there are mature trees in your yard whose roots could have cracked the pipe in their search for moisture.

● Water Heater: Drain water until it is clear of sediment; test temperature pressure relief valve, clean burner and ports (gas heater).

● HVAC System: Have a professional tune up your heat/air conditioning system.

● Carpets Professionally cleaned: Get rid of all those dust mites, and deep dirt you can’t get to yourself.

● Air Ducts cleaned: Depending on if you have allergies in the family, have a professional clean out the ducts every 1 to 2 years.